Service provider part

The Idea from design service provider is support many customers with easy way and easy trouble shooting .

So that we use

( 1 )  MPLS to provide the following

- Assigns labels to each data packet, controlling the path the packet follows. MPLS greatly improves the speed of traffic, so users don't experience downtime when connected to the network.

- Virtual routing and forwarding provides a path to configuring multiple routing instances on either a router or Layer 3 switch. The purpose is to keep customer traffic and routing separate but through the same hardware.

( 2 ) BGP to provide the following

- The PEs in the provider network using MPLS BGP use the Multiprotocol-Border Gateway Protocol (MP-BGP) to dynamically communicate with each other.

- To carry MPLS labels between routers .

( 3 ) OSPF as dynamic routing protocol

First ( MPLS usage )

In service provider network we use new address family,

it called ( VPNv4 )

VPNv4  consists of two addresses

- Route distinguisher to differentiate customer networks  .                ( RD _ 96 bit )

- Route target to vrfs ( RT _ 64 bit )

Second ( MP_BGP usage )

- In the design we used Route reflector routers ( P1 and P2 ) to support redundancy, availability and reduce number of BGP sessions between routers .

- MP_BGP carries RD and RT attributes .

Third ( OSPF )

To exchange network between routers

( Basic configuration to VRF ) Cairo branch

ip vrf CIB

rd 1:1

route-target 1:1

int loo 0

ip add 50.50.50.50 255.255.255.255

ip ospf 1 area 0

int loo 1

ip vrf forwarding CIB

ip add 40.40.40.40 255.255.255.255

int f0/1

ip vrf forwarding CIB

ip add 2.0.0.2 255.255.255.252

no sh

int f1/1

no swit

ip vrf forwarding CIB

ip add 1.0.0.2 255.255.255.252

no sh

( Basic configuration to  mp_bgp )

router bgp 200

neighbor 100.100.100.100  remote-as 200

neighbor 100.100.100.100 update-source Loopback0

neighbor 200.200.200.200  remote-as 200

neighbor 200.200.200.200 update-source Loopback0

address-family vpnv4 unicast

neighbor 100.100.100.100 activate

neighbor 100.100.100.100 send-community extended

neighbor 200.200.200.200 activate

neighbor 200.200.200.200 send-community extended

address-family ipv4 vrf CIB

neighbor 10.10.10.10 remote-as 100

neighbor 10.10.10.10 activate

neighbor 10.10.10.10 update-source Loopback1

neighbor 10.10.10.10 ebgp-multihop

note

( the same configuration on Alex and Aswan with difference in IPS )